Research Project Documentation

AI-Derived Acquisition Standards for Diffusion MRI (DWI) and Data Birth Integrity (DBI)

# 1. Project Title

AI-Derived Acquisition Contracts for Diffusion MRI: Improving Biomarker Stability and Model Generalization through Data Birth Integrity

# 2. Executive Summary

Diffusion-weighted imaging (DWI) is widely used in neuroscience and clinical research, yet suffers from high acquisition variability, leading to:

Poor reproducibility

Limited biomarker reliability

Reduced cross-site generalization

Scarcity of reusable public datasets

This project proposes a new framework:

Deriving acquisition-level standards from the structural requirements of AI and automation systems.

We introduce:

AI-Ready Acquisition Schema (AI-RAS) for DWI

Data Birth Integrity (DBI) as a measurable structural quality metric

A validation framework to enforce acquisition constraints

An empirical evaluation of impact on biomarkers and AI models

The project will be implemented at an imaging center and evaluated through controlled experiments.

# 3. Problem Statement

Current DWI datasets exhibit:

Inconsistent b-values

Variable gradient directions

Missing or incomplete metadata

Scanner-dependent variability

Protocol drift across time and sites

These inconsistencies:

Increase harmonization burden

Reduce reproducibility

Introduce bias into AI models

Limit dataset reuse

Existing standards such as:

DICOM

Brain Imaging Data Structure

do not enforce acquisition-level consistency.

# 4. Research Hypothesis

Enforcing AI-derived acquisition constraints at data creation improves diffusion biomarker reproducibility and machine learning generalization compared to standard post-hoc harmonization approaches.

# 5. Conceptual Framework

## 5.1 Data Generation Pipeline

Reality → Measurement → Encoding → Dataset → AI Model

Errors introduced at the measurement and encoding stages propagate downstream.

## 5.2 Key Definitions

Data Birth Integrity (DBI). A quantitative measure of structural correctness at data creation.

AI-Ready Acquisition Schema (AI-RAS). A set of acquisition constraints derived from AI pipeline requirements.

Acquisition Contract. A versioned specification defining required acquisition conditions.

Protocol Drift Index (PDI). A measure of deviation from defined acquisition protocol across time or sites.

# 6. AI-Derived DWI Acquisition Standards

These standards are derived from requirements of diffusion modeling, tractography, and machine learning pipelines.

## 6.1 Multi-Shell Requirement

Must include:

b = 0

≥1 diffusion shell ≥1000

Prefer multi-shell acquisition (e.g., 0, 1000, 2000)

## 6.2 Gradient Direction Requirement

Minimum:

≥30 directions (single-shell)

≥60 directions (multi-shell)

## 6.3 Gradient Table Integrity

Mandatory presence of:

bvals

bvecs

Alignment and consistency checks required.

## 6.4 Spatial Resolution Constraint

Target voxel size: ~2 mm isotropic (acceptable range defined).

## 6.5 Phase Encoding Metadata

Required fields:

PhaseEncodingDirection

TotalReadoutTime

Fieldmap availability (if applicable)

## 6.6 Protocol Versioning

Each acquisition must include: protocol_version = "DWI_V1.0"

## 6.7 Naming Consistency

Example:

sub-001_ses-01_dwi.nii.gz

sub-001_ses-01_dwi.bval

sub-001_ses-01_dwi.bvec

## 6.8 Mandatory Metadata Fields

b-values

gradient directions

voxel size

TE/TR

scanner vendor

field strength

# 7. Data Birth Integrity (DBI) Metric

## 7.1 DBI Components

| Component | Description |
| --- | --- |
| Metadata completeness | Required fields present |
| Protocol conformity | Matches acquisition contract |
| Gradient integrity | Valid diffusion encoding |
| Spatial consistency | Voxel size within range |
| Naming compliance | Standardized naming |
| Drift control | Protocol adherence over time |

## 7.2 Example DBI Formula

DBI = w1*M + w2*P + w3*G + w4*S + w5*N + w6*D

Where:

M = metadata completeness

P = protocol conformity

G = gradient integrity

S = spatial consistency

N = naming compliance

D = drift adherence

# 8. Methodology

## 8.1 Study Design

Two conditions:

| Condition A | Condition B |
| --- | --- |
| Standard acquisition | AI-RAS enforced acquisition |

## 8.2 Data Collection

Retrospective dataset (baseline)

Prospective dataset (after enforcement)

## 8.3 Metrics

Structural Metrics:

% missing metadata

Naming inconsistency rate

DBI score

Operational Metrics:

Cleaning time (hours)

Pipeline failure rate

Reprocessing frequency

AI Metrics:

Model AUC

Cross-site performance variance

Calibration stability

Biomarker Metrics:

FA reproducibility

MD stability

Tractography consistency

## 8.4 Statistical Analysis

t-tests for group comparison

Variance analysis

Effect size calculation

Correlation between DBI and model performance

# 9. Implementation Plan

## Phase 1 — Baseline Analysis

Analyze historical DWI datasets

Compute DBI and PDI

## Phase 2 — Schema Definition

Define AI-RAS for DWI

Implement validation rules

## Phase 3 — Pilot Deployment

Apply at imaging center

Introduce validation at export stage

## Phase 4 — Evaluation

Compare before vs after

Analyze metrics

# 10. Expected Outcomes

Structural Improvements:

Increased metadata completeness

Reduced protocol drift

Operational Improvements:

Reduced cleaning time

Fewer pipeline failures

AI Improvements:

Reduced model variance

Improved generalization

Scientific Impact:

More reproducible biomarkers

Increased dataset reusability

# 11. Potential Contributions

Scientific Contributions:

Formal definition of DBI

Empirical evidence linking acquisition structure to AI performance

Framework for acquisition-level standardization

Technical Contributions:

AI-derived acquisition schema

Validation framework

Drift monitoring approach

Practical Contributions:

Improved imaging workflows

Reduced harmonization burden

Better dataset sharing

# 12. Limitations

Limited control over scanner hardware

Site-specific variability

Adoption resistance

Need for multi-site validation

# 13. Future Work

Extend to other imaging modalities

Multi-site validation studies

Integration with clinical data models (e.g., OMOP Common Data Model)

Expansion into certification framework

# 14. Broader Impact

This work contributes to:

Reproducible science

Data-centric AI

FAIR data principles

Global data accessibility

# Final Statement

This project reframes standardization: from post-hoc harmonization to preemptive structural enforcement at data creation.

By grounding acquisition standards in AI requirements, this work aims to:

Improve reliability of scientific data

Enable scalable AI systems

Reduce structural inefficiencies in data pipelines